Research on AI-Assisted Design Flow for 4KB SRAM using OpenRAM and SKY130 PDK

Kaavya Kamat

Abstract — This report establishes a framework for designing a 4KB (1024×32) SRAM macro using the OpenRAM compiler and the open-source SKY130 PDK. Operating at 1.8 V with a target access time under 2.5 ns, the workflow leverages AI-assisted prompt engineering to translate complex compiler automation into discrete, verifiable sub-blocks. The document outlines structural architecture, sub-block operations, and a guide for OpenRAM error resolution.

Keywords — SRAM, Memory, SKY130, OpenRAM, ASIC Design, VLSI.

I. Introduction

A 4KB SRAM is a small, ultra-fast volatile memory that stores 4,096 bytes of data using a transistor-based flip-flop structure. Unlike DRAM, it doesn't need constant refreshing, allowing near-instantaneous read/write speeds as long as power is maintained. Due to its speed and small footprint, it is commonly used for internal registers, CPU cache, or runtime variable storage in low-power microcontrollers.

II. SRAM Architecture

The core storage mechanism is the standard 6T CMOS cell consisting of two cross-coupled inverters forming a bi-stable latch, and two NMOS access transistors controlled by the wordline (WL).

- In the hold state, WL is held at 0V to isolate the cross-coupled inverters, which maintain their state via continuous feedback.

- During a read, BL and BL’ are precharged to 1.8V by a PMOS-trio precharge circuit, and WL is raised to 1.8V; the ‘0’ node pulls its bitline down to create a small differential voltage ΔV. To prevent accidental flips, the cell requires high Read Stability (SNM), meaning a high Cell Ratio where the pull-down NMOS is stronger than the access NMOS.

- A write operation drives WL high to force a node to ‘0’—the primary write mechanism, since NMOS passes a strong ‘0’ but a weak ‘1’—requiring a high Write Factor so the access transistor can overpower the pull-up PMOS.

- A high-gain Sense Amplifier detects the minimal bitline swing (ΔV ~100–200 mV) and rapidly amplifies it to full logic levels to keep access times under 2.5 ns, while a strong Write Driver uses tri-state buffers to forcefully pull either BL or BL’ to 0V to override the cell’s internal latch.

III. OpenRAM Architecture and Sky130

OpenRAM is a Python-based memory compiler that takes a high-level configuration file and synthesizes custom SRAM blocks by tiling pre-designed, pitch-matched leaf cells. When targeting the SkyWater 130nm (SKY130) open-source PDK, OpenRAM utilizes cell libraries strictly calibrated to SKY130 design rules and transistor models, enabling silicon-ready, open-source hardware generation. It relies on four main representation files for each leaf cell (such as the 6T bitcell, sense amp, and decoders):

- GDSII (.gds) – Physical layout geometries for SKY130 manufacturing.

- LEF (.lef) – Abstract boundaries for ASIC flows like OpenLane.

- SPICE (.sp) – Transistor-level netlists using SKY130 models for verification.

- Liberty (.lib) – Timing/power models.

IV. Setup and Debugging

Setting up this design flow involves two main steps, linking the tool folders to your system paths, and configuring your chip dimensions (1024×32) in a Python script. If the tool crashes, look for three common fixes:

- Path Errors – ensure the terminal knows where OpenRAM is installed by updating the .bashrc file

- Spacing Errors (DRC) – if layout lines are too close together, adjust the metal rules in the configuration file

- Simulation Errors – if the simulation program (ngspice) freezes during testing, tweak its accuracy settings to help it pass.

V. Pre- and Post-Layout Verification

Before generating the physical layout, ideal pre-layout simulations are run in ngspice to verify that the core architecture can meet the target access time of under 2.5 ns. After OpenRAM dynamically compiles the physical wiring, post-layout verification extracts real-world parasitic resistance and capacitance from the metal lines. These extra factors slow down signals, making post-layout checks crucial to ensure the final chip operates reliably at 1.8 V

V. Summary

In summary, this research establishes a comprehensive framework for OpenRAM-based memory generation leveraging the SKY130 process node. By decoupling the core SRAM architecture into verifiable sub-blocks and defining the structural toolchain dependencies, this foundational work directly supports the subsequent compilation, characterization, and physical verification phases of the project.

VI. References

[1]  SkyWater Technology Foundry, “SKY130 Open Source PDK.”

[2]  Shon Taware, “SRAM SKY130 Repository,” GitHub.

[3] OpenRAM Project, “OpenRAM Memory Compiler Documentation and Source Code.”